# NAG 5

## TIKOKINO SCHOOL

## HEALTH AND SAFETY

### PROCEDURE – Water Testing

GUIDELINES:

- A minimum of 11 monitoring samples are required to be taken over the school year (11 months) on 2 different days of the week over each quarter, with a maximum of 45 days in between samples.

- Testing will be completed by the approved laboratory of:

Water Testing Hawkes Bay Ltd, 500 Williams Street, Mahora, Hastings, 4120 			(ph 06 8706449; email: info@watertestinghb.co.nz)

- Testing needs to be completed for presence / absence of coliforms, faecal and total

- Results of tests to be retained by the school

- Where excess coliforms are present, action should be taken under advice from the Health Protection Officer – 06) 834 1815.

- Process for obtaining sample is as follows:

  - A different tap is to be used with each sample

  - Using a clean cloth or paper the outlet is cleaned to remove any dirt

  - The tap is to be run for at least 1 minute prior to taking the sample

  - The supplied container is to be filled leaving an air space of 2 to 3 cm at the top of the bottle for the sample to be mixed in the laboratory

  - The container is then sealed and the packed in the ice packs supplied in the chilly bin.

    - Ice packs can be stored dry

    - Prior to use, soak the ice pack in water to swell, then freeze.

e.   The chilly bin containing the sample is delivered to NZ Laboratory and the chilly bin is then returned, by the tester, to the school.